# Transfer Learning Training with EEG for Categorization of Movement-Related Brain Activity (2026)

Denstas Panagiotis, Uniwa, bme (AM:21388015), Supervised by Kakko Ioannh, Machine learning

Abstract— This study evaluates Transfer Learning (TL) as a solution for classifying EEG motor imagery and actual movement data under "small sample size" constraints. Using a shallow 1D CNN architecture designed to combat overfitting through spatial filtering and weight sharing, a base model was trained on motor imagery and then adapted for real movement tasks. Preprocessing involved strategic temporal trimming and 1% outlier clipping to stabilize normalization. Results showed that the transfer model successfully exceeded the base model’s performance, achieving a 56% validation accuracy. Findings demonstrate that TL effectively locks in stable neural features, allowing high-capacity classification layers to generalize even when data is scarce, and signal-to-noise ratios are low.

Index Terms—EEG, Transfer learning, deep learning, small sample size, ERP.

# I. Introduction

E

EG data presents many problems when trying to train a machine learning model, especially when we have a very small number of samples. EEG data is characterized by an extremely unfavorable combination of high dimensionality (many channels and time points) and a low signal-to-noise ratio (strong presence of artifacts and interference), a fact that is dramatically exacerbated when the training set is small. Due to the inherent non-stationarity of the brain signal (it changes over time) and high inter-subject variability (from person to person), machine learning models struggle to identify stable patterns. This often leads to overfitting: the models "memorize" the noise of the few available samples instead of learning generalizable brain functions, making them unreliable for new, unseen data.

Creating these models is particularly useful for the development of innovative technologies.

The study used the data from an event-related electroencephalography (EEG) experimental design, aiming to investigate brain activity related to the actual and imagined execution of lower limb movements under visual stimuli. Time-synchronized markers (triggers) allowed for the study of movement-related cortical potentials and spectral changes.

The training of the base model was made using imaginary movement data and after that the transfer learning was made using real movement data. The inversed was also tried to see the results, additionally the are trials were  the algorithm was trained with different time frames. The structure of the base model was created with trial and error trying to get the based results

# II. Experiment

EEG data were recorded at a sampling rate of 512 Hz using 72 electrodes. Of these, 64 electrodes were used for EEG recording according to the International 10–20 System. Vertical electrooculograms (VEOG) were recorded from electrodes 65–66, and horizontal electrooculograms (HEOG) from electrodes 67–68. Electromyographic activity (EMG) was recorded from electrodes placed on the lower limbs (electrodes 69–72).In the training of the model only the 62(out of the 64 electrodes of the EEG was used as two physical positions are reserved for the Reference and Ground electrodes, which are essential for recording but do not provide independent brain signal data.

Two experimental conditions were used in a predefined order: Actual movement with visual stimuli,Motor imagenery (mental movement) with visual stimuli.

Each condition included 80 trials, consisting of 20 trials of right lower limb movement, 20 left, 20 bilateral movement, and 20 trials with no movement. Each trial lasted 3 seconds, followed by an inter-trial interval of 1.5 seconds. All experimental events were marked with digital time triggers, allowing for precise temporal synchronization of EEG and EMG.

Distinct trigger codes were used to identify the type of movement (right, left, bilateral, none), the execution mode (actual or mental), and the sensory modality of the stimulus (visual). Additionally, markers were used for the onset and offset of the movement period, allowing for the segmentation of data into task-related time windows (epochs).

# III. Data preparation

The First step in training the model was the creation of data in a correct format.

Using EEGLAB (MATLAB) the samples were filtered to use only the useful frequencies, and the sampling rate was reduced from 512 Hz to 254 samples per second in order to save computational resources. Subsequently, segmentation (epoching) was performed so that each stimulus (trigger) was isolated, covering a time window from 0.3 seconds before the event to 3 seconds after the event. The data were then exported from MATLAB in CSV format.

Figure 1: Frequency spectrum of the data that was used for training.

The data were loaded into Python individually and combined in the following shape/format: subject, samples (actual numbers), and channels in order to be used in a CNN.

In order to cut signals from electrodes that were disconnected and potential eyeblink that had extremely high values, the 1% of the biggest values was clipped. This method is not perfect, and the results would be better if we were able to cut those signals all together. The aim was to lower the ability of the model to detect those really high values that didn't correspond to useful information, while also making the normalization better.

If we didn't cut the std would have high values crushing the small signals in the normalization.

After that, the data was normalized using the standard method, subtracting the mean and dividing the standard deviation.

They were split to the data that will be used for training and the data that will be used as a test in 8:2 ratio. This process was used in both groups (real and imaginary).

IV. Base model

The base model training was particularly challenging, and the end result was not really satisfactory. Many different methodologies were tried, most of which ending up making the model perform worse.

The big problem was overfitting. In simple terms the model was learning the individual saples and not learning to classify something that it has never seen before. That problem is even bigger when the sample size is small, like in this case. The main challenge was creating a model smart enough to understand the pattern but not able to memorize the individual data. In order to do that, a shallow model using 1dconvolution was used.

When data is scarce, 1D CNNs are superior because they use weight sharing to drastically reduce the number of parameters the model must learn, preventing the "memorization" common in Dense networks. By using a sliding window to detect local patterns, the CNN gains translation invariance, meaning it can recognize a critical signal feature regardless of where it appears in the timeline. This focused "inductive bias" allows the model to extract meaningful features from just a few hundred samples, whereas a larger, fully connected architecture would likely fail to generalize.[4]

In the start of the model the time frame is selected. The best results came from cutting 0.3 seconds at the start and ending 1 second after. This time frame was selected as a base as it produced the best result (the original sample was from 0.3 seconds before the trigger to 3 seconds after).

In neurophysiological research, particularly when analyzing Ready-Period-Evoked (RPE) potentials, the epoching window is strategically split to capture the full spectrum of the neural response while ensuring data integrity. The 0.3-second pre-trigger interval serves as a vital baseline correction period; by measuring the brain’s "resting" electrical state immediately before the stimulus, researchers can subtract this mean value from the subsequent signal to eliminate DC offsets and non-task-related noise. Conversely, the 3-second post-trigger window is extended to accommodate Slow Cortical Potentials (SCPs) and late-stage cognitive processing, such as the Readiness Potential or the P300 wave, which often persist long after the initial sensory onset. This longer duration accounts for variability in human reaction times and ensures that the signal does not suffer from "edge artifacts" during digital filtering, allowing the waveform to return to a stable equilibrium and providing a complete picture of the transition from preparation to execution and recovery.

Figures 2-3: Results of data with no time trimming base model (imaginary movement data) and after transfer learning (real movement data). The results are worse when comparing this with the final trimmed version that will be shown later.

It is possible to use the data that was cut but in this case the artifacts it contained were too much and over exaggerated the over fitting, while also making the most useful signal relatively smaller.

Figure 4: Architecture of the model, the layers inside the red box were removed later in the transfer learning.

This model uses a Temporal Convolution design to handle complex brain signals. It starts with a "bottleneck" layer that looks at all 62 electrodes at once but ignores time for a split second. This step learns how to mix and match the raw signals to create 8 clean "Spatial Patterns," effectively filtering out background noise from the scalp before moving on to the actual data analysis.

Next, the model moves into Temporal Filtering, where it finally analyzes how the signals change over time. By looking at 32-sample windows (about 125ms of data), it can "hear" the specific rhythms of the brain, such as those used when you imagine moving your hand. It uses Average Pooling to smooth out jagged signal spikes and Spatial Dropout to make sure it doesn't get too focused on just one specific pattern, keeping the model flexible.

The third stage is a Refinement Block that zooms in closer. It uses a smaller window (16 samples) to catch quick, complex bursts of brain activity that the previous layer might have missed. At this stage, the model also uses L2 Regularization, which acts like a filter to stop the AI from "memorizing" the unique noise of one person, helping it work better for different users.

Finally, the model enters the Classification Head. It takes all the refined patterns it found and flattens them into a long list of 256 values. These are fed into a final "Dense" layer that acts like a judge, weighing all the evidence to decide which of the 4 categories the brain activity belongs to. It uses a "MaxNorm" rule to keep its internal math from getting too extreme, ensuring the final prediction is stable and reliable.

IV. Tranfer model

Figure 5: Architecture of the transfer model

This model uses Transfer Learning, a technique where the model we have already created, that already understands EEG patterns are used as a foundation for a more powerful classification engine.

By "freezing" the original 1D CNN, the system locks in the spatial and temporal filters it has already learned and removed the last 3 layers of the base model.

This allows the base model to act as a specialized Feature Extractor, turning raw, messy brain signals into a polished summary without having to relearn the basics of brain rhythms from scratch.

To ensure the new parts of the model work correctly with the old ones, the data first passes through a Batch Normalization layer. This acts as a "clean-up" step, re-centering and scaling the data coming out of the frozen model. This is critical in transfer learning because it ensures the new layers receive a consistent signal, which fixes common training issues and helps the model reach a higher accuracy much faster.

The model then moves into a High-Capacity Processing stage. While the original model was very lightweight, this version adds two massive layers of 1,024 and 512 neurons.

This provides the "brainpower" needed to look at the 256 extracted features and find incredibly subtle differences between them. By expanding the data this way, the model can better separate the four categories, such as distinguishing between very similar motor imagery tasks. This is possible to do (with no major over fitting) because the head (base model) has already been built and cannot learn to really on certain artifacts to categorize the data.

To keep this extra power under control, the architecture uses a 30% Dropout rate. This prevents the model from simply memorizing the training examples from every feature they have. Forcing the network to work around "missing" neurons during training, it becomes more robust. The process ends with a final SoftMax layer that translates all this deep processing into a clear percentage of probability for each of the four possible classes.

IV. Results and graphs

Base model training graphs

Figures 6-9: Base Model Training (imaginary data). The blue line shows the accuracy from the data the that we trained the model on while the red line, validation accuracy, shown the accuracy of the external model

The model was able to reach a validation accuracy of over 50 percent, while in certain instancy's it could jump to 54 percent.

As expected, the best performance was saved and used for the training of the transfer model.

Tables of classification of the base model

Figures 10-13: base model tables of classification

An unexpected result of the classification was the model's ability to recognize the left foot trails, in most cases even better than the trails of nothing.

While on the other hand intuitively it was expected that the model will have a much easier time recognizing the nothing trials in comparison with everything else. (A small study trying to find the root cause of this could be interesting, even if it is caused by the model been unrefined).

Transfer model training graphs

Figures 14-16: Transfer model training (real movement data). The blue line shows the shows the accuracy from the data the that we trained the model on while the red line, validation accuracy, shown the accuracy of the external model (the base model of this was 51%)

The results are exceptionally good, usually been able to reach 56%.It is uncommon for transfer learning to been able to exceed the validation of the base model, but in this case it did happen. It seems that the quality of the erg was probably better in terms of quality and this was tested later

This shows great potential if a better base model is able to be created. Even at the first epoch, the model easily exceeded 45% validation accuracy.

Also, not worthy is that the validation accuracy is higher than the actual accuracy due to multiple factors:1. heavy drop out of the base model and spatial drop out of the transfer model, that did not play a role in the validation accuracy

2.Because the base model been frozen and on top of the transfer model it was hard for it to learn the artifacts and been able to categorize the data from them.

Tables of classification of the Transfer model

Figures 17-18: Transfer model tables of classification

The results are very from the base model and do not seem very consistent, so no major conclusion was drawn. This model struggled a bit to detect trails for both legs, which makes logical sense.

V. Inverse training experiment

The better results from the transfer in comparison to the base model seem to suggest that the data from the real movement was better than the imaginary.

So, as a further experiment the base model, even if its architecture was built for imaginary moment classification) was trained on the real movement data.

Image 17-18: Training of the base model using the real movement data and the transfer model with imaginary movement data.

As expected, the base model easily reached 58 validation accuracy, confirming the data is higher quality.

When doing the transfer learning, the model reached 48 validation accuracy which makes logical sense.

Figures 19-20: Tables of the base model using the real movement data and the transfer model with imaginary movement data.

The categorizing of the base model in this case was better of classing nothing, and we do not see the left foot trail accuracy exceed the other like the base model trained on imaginary data like before. This high left foot accuracy returns after the transfer of learning with the imaginary data. Indicating that for certain reasons, the imaginary movement of the left foot differs more than the others.

VI. Conclusion and Discission

The training of the base model was a troublesome process to the small size and probably bad quality of the EEG data, been able to reach 54% validation accuracy. The models were prevalent to over fitting, and multiple measures were needed to combat this tendency.

The transfer learning was relatively successful and been able to exceed the base model. Showing immense potential for transfer learning even with all the limitations this small study had. The most interesting result was the model ability to classify the imaginary movement of the left foot, even from nothing

The best way to improve the results would be the inclusion of more data, but the collection of which is hard. The probable next steps could be to filter the data artifact in different ways to prevent overfitting. Data augmentation (with noise and time shift) and frequency filtering were tried but were not included as they were unsuccessful.

An interesting application with practical sense would be, to train the base model using all the data in a data base and using transfer learning on the samples of a particular person

It should also be noted that the computational power available for testing different models was relatively low, which, due to time constraints, did not allow for the testing of numerous models or different optimization methods to improve the results.

# VII. Sources

[1]Lotte, F., et al. (2018). "A review of classification algorithms for EEG-based brain–computer interfaces: a 10 year update". Journal of Neural Engineering.

[2]Lawhern, V. J., et al. (2018). "EEGNet: a compact convolutional neural network for EEG-based brain–computer interfaces". Journal of Neural Engineering.

[3]Urigüen, J. A., & Garcia-Zapirain, B. (2015). "EEG artifact removal—state-of-the-art and guidelines". Journal of Neural Engineering.

[4]Kiranyaz, S., et al. (2021). 1D convolutional neural networks and applications: A survey.

Corresponding author: I.Kakkos (e-mail: author@lamar.colostate.edu).

K. Raktivan, I. Kakkos, I. Vezakis and G.K. Matsopoulos are with the

g